# Plan: Relative Distance Measurement Between Devices

## Goal

Calculate the relative distance between two ESP32 devices in real time.

## Options Evaluated

### 1. RSSI (No extra hardware)

- Use info->rx_ctrl->rssi from ESP-NOW receive callback

- Estimate distance via log-distance path loss model: distance = 10 ^ ((measuredPower - rssi) / (10 * n))

  - measuredPower: RSSI at 1m (calibrate per environment, typically -40 to -50 dBm)

  - n: path loss exponent (2.0 free space, 2.5-4.0 indoors)

- Pros: zero extra cost, already available

- Cons: noisy (+-5m), affected by obstacles/orientation/environment

- Viable as a rough proximity indicator (near/mid/far zones), not precise measurement

### 2. UWB - Ultra-Wideband (Recommended for accuracy)

- Module: DWM1000 or DWM3000 (SPI interface)

- Accuracy: ~10cm using Two-Way Ranging (TWR)

- One device acts as anchor, the other as tag (or symmetric TWR)

- Library: arduino-dw1000 or Decawave’s own SDK

- Pros: cm-level accuracy, works well for V2V range

- Cons: extra module (~$5-15 each), SPI wiring, more code complexity

### 3. GPS (Good for longer range)

- Module: NEO-6M or NEO-M8N (UART interface)

- Each device reads its own GPS coordinates and broadcasts them via ESP-NOW

- Receiver computes Haversine distance

- Accuracy: 2-5m (civilian GPS)

- Pros: works at any range, absolute positioning

- Cons: extra module, no indoor use, slow update rate (~1Hz), 2-5m error

### 4. LiDAR/ToF (Short range only)

- Module: VL53L0X (I2C)

- Accuracy: ~1cm, but max range ~2m

- Not suitable for vehicle-to-vehicle distances

## Recommendation

- For a quick prototype: implement RSSI-based zones (close/medium/far)

- For production accuracy: add UWB modules (DWM1000)

- For outdoor long-range: add GPS modules

## Implementation Steps (RSSI - quickest path)

- In onDataRecv, extract RSSI: int rssi = info->rx_ctrl->rssi;

- Add calibration constants (measuredPower, n) at the top of the file

- Write float estimateDistance(int rssi) using the log-distance formula

- Optionally smooth with a moving average filter (5-10 samples) to reduce noise

- Print/broadcast the estimated distance alongside the motion state

## Implementation Steps (UWB - accurate path)

- Wire DWM1000 to each ESP32 via SPI

- Install arduino-dw1000 library

- Implement TWR (Two-Way Ranging) protocol between the two devices

- Read distance from the ranging result

- Integrate with existing ESP-NOW data (send distance alongside acceleration)